Nmap Scan on Windows 7

I turned off windows firewall from settings for both home and public networks.

I used this ping command to check if kali machine is able to communicate with windows machine.

This is a host discovery scan. It tells me the host is live. Gives me the MAC address.

These are the ports that are currently opened

Version detection for each port thats opened

Operating system detection command was run.

I run this command to check if port 445 has MS17-010 vulnerability. It checks if the flaw is present.

Results: vulnerable to MS17-010. The risk factor is high.

I used this command to check windows against a library of known vulnerabilities

- Vulnerable to ms17-010

- Not vulnerable to ms10-054

- Access denied for ms10-061

- Access denied for cve-2012-1182

This command helped with finding out the operating system and device name. Anonymous access read is only applicable for IPC$. Not for both ADMIN$ and C$.

This command tells me that port 3389 is closed and remote desktop protocol isn’t enabled on windows.

Zenmap Intense Scan for IP 192.168.0.116

Exploitation of MS17-010 Vulnerability in Windows 7 using Metasploit

Running msfconsole command to open Metasploit toolbox

This command tells Metasploit to use the tool specifically to exploit MS17-010 vulnerability

This command points Metasploit to windows 7

IP address Kali: 192.168.0.122

IP address windows: 192.168.0.111

This command is used to tell Metasploit where to send back the connection (local host)

Prints out everything Metasploit has configured

RHOSTS and LHOST is visible RPORT 445 SMB port is physically set as well

Payload package creates the remote connection after the exploit

I used run command to actually start the exploit and here are the results

Meterpreter indicates that im inside Windows 7. I have a live shell running with full system level access on Windows 7 machine from my Kali terminal

From getuid, i can check if i have the highest level of access. Successfully exploited the vulnerability and gained full administrative control.

Through sysinfo, i can show proof of the machine im inside

This is the list of running processes in hacked machine

I am inside a live Windows CMD on the hacked machine and the desktop folder is empty

Shows password hashes from compromised windows 7 machine using the hashdump command. These hashes represent encrypted local account passwords of administrator, guest, and win. They could be cracked offline and used in further attacks.

Defense:

- Install the MS17-010 patch (KB4013389). It fixes the root cause directly

- Disable SMBv1, since it's outdated and no longer needed

- Block port 445 from untrusted networks using a firewall

- Segment the network (VLANs) so a compromised machine can't easily reach others

- Enable SMB signing to add protection even on older systems

- Enforce strong local passwords, so extracted hashes are harder to crack

- Use monitoring/IDS tools to detect known EternalBlue attack patterns

- Retire Windows 7, since it no longer receives security updates at all